# SECTION 14 46 00

# VERTICAL RECIPROCATING CONVEYOR

PART 1   GENERAL

  - SECTION INCLUDES

- Design, fabrication, and installation of one (1) vertical reciprocating conveyor (VRC) including drive unit, manual controls, gates, and enclosures as shown on project drawings and as specified herein.

  - RELATED SECTIONS

- Section 03300:  Cast in place concrete.

- Sections of Division 16:  Electrical as applicable.

  - REFERENCES

- ASME – American Society of Mechanical Engineers (ASME B20.1).

- AWS – American Welding Society.

- NEMA – National Electrical Manufacturer’s Association

  - SUBMITTALS

- Product Data:  Submit latest edition of VRC data sheet and outline drawing with the proposal.

- Shop Drawings:

- Submit General Arrangement Drawing for approval within 3 weeks after receipt of an order, including plans, elevations, sections of the VRC, base plate and lateral loading values, and recommended pit dimensions if applicable.

- Submit VRC Specification Sheet for approval within three weeks of receipt of an order, including scope of work, operating and control voltages, lift speed, type of paint, and any special project notes.

- Closeout Submittals provided with equipment:

  - Schematic Drawing including control panel layout and Bill of Materials reflecting original manufactured part numbers.

  - Installation Manual and Electrical Installation Guide.

  - Owner’s Manual including spare parts list, exploded parts drawings, operating instructions, maintenance schedule, service, and troubleshooting guidelines.

  - QUALITY ASSURANCE

- Manufacturer must have a minimum of five (5) years experience in the manufacture of vertical reciprocating conveyors.

- All structural welding performed by manufacturer must be done by welders certified to AWSD.1.1.

- Manufacturer must guarantee compliance with ASME B20.1 Safety Standard for Conveyors and Related Equipment.  Manufacturer further guarantees to repurchase the equipment at full purchase price in the event that Owner is unable to use the equipment due to lack of elevator code compliance.

- Installer shall have the approval of the manufacturer and have a minimum of five (5) years experience in the installation of vertical reciprocating conveyors.

  - WARRANTY

- The manufacturer shall warrant the VRC free of manufacturing defects beginning (30) days after shipment with the following minimums:

- PFlow components – one (1) year parts and labor.

- Purchased components – one (1) year parts, ninety (90) days labor.

PART 2 – PRODUCTS

2.01	MANUFACTURER

- Series B VRC manufactured by PFlow Industries, Inc., 6720 N. Teutonia Ave, Milwaukee, WI  53209  
Phone (414) 352-9000    Fax (414) 352-9002    e-mail info@pflow.com    website: www.pflow.com

2.02	VRC MECHANICAL SPECIFICATION

- Structure:  Stackable, modular formed steel panel frame.

- Capacity:  The VRC shall be rated at a live load capacity of 300 lbs. (500 lbs. is maximum)

- Speed: The VRC shall have a minimum lifting speed of 18 feet per minute when loaded to capacity. (36 FPM is the maximum)

- Vertical Travel:  The VRC shall have a maximum lift height of 17’-3” with a total of 2 operating levels with a maximum distance between floors of 14’-9”.  Platform load height shall be at a height of 30” from finished floor at both levels.  Actual finished floor-to-floor distance is         .

- Lift Platform:  The VRC platform shall provide a usable space of 36 inches wide x 36 inches long x 36” load height with a steel deck plate surface.

- Safety Enclosure:  The VRC shall be completely enclosed with solid panels on all four sides.  The footprint of the equipment shall be no greater than 48” x 48” excluding control stations.

- Motor Location:  The gearmotor shall be located at the base of the VRC with maintenance access through a removable panel.

- Deflection Under Load:  When loaded to rated capacity, no portion of the VRC shall exhibit permanent deformations.

- Lifting Means:  Raising and lowering of the carriage shall be provided by roller chains over sprockets with common drive shaft.

- Broken Chain Safetys:  Dual spring–loaded safety pins shall be mounted on the platform and connected directly to the lifting chains.  Pins prevent uncontrolled carriage descent in case of chain failure.  Activation of pins shall prevent the platform from falling more than 6”.

- Access Gates:  Gates are required on all operating sides of the VRC at each level of operation.

  - The gates shall be swing type that have field reversible swing for right-hand or left-hand opening.

2.	The gate locations (load pattern) shall be field relocatable by nature of the VRC’s   modular design.

3.	Each gate must be equipped with an electro-mechanical interlock to prevent opening of the gate unless the carriage is present, and to prevent operation of the VRC unless all gates are closed.

- Signs:  “NO RIDER” signs shall be provided.  Lettering shall be a minimum of 2” high for visibility.

2.03	VRC ELECTRICAL SPECIFICATION

- Electric Motor:

  - Motor shall be a minimum of 1 horsepower for 300 lbs. or 500 lbs. at 18 FPM and 1½ horsepower for 500 lbs. at 36 FPM.

  - All motors are three phase and shall be designed for continuous duty at ambient temperatures from 32° to 102° Fahrenheit.

  - The motor shall not automatically restart when the overload device is reset.

  - The motor shall be equipped with a heavy-duty, fast-acting, fail-safe industrial brake to ensure the brake will hold in case of power failure.

- Controls:

  - Each operating floor level shall be equipped with a momentary contact push button control station with call, send, and mushroom style E-stop operators for manual control of lift operation.

  - An internally pre-wired NEMA 12 rated main control panel located within the base of the lift structure shall be provided.  The control panel contains terminations for field devices and relay logic for control of the lift.

  - Travel Switches: The VRC shall utilize electrical switches to position the carriage at the upper and lower levels, and shall include an over-travel limit switch to stop the platform should the upper floor switch fail.  Under-travel is protected by physical stops.

  - Chain Tension Safety Device: Monitors each chain and shuts off motor while engaging brake in the event of a slack or broken chain.

- Wiring:

  - The VRC shall ship pre-wired utilizing industrial duty IP67 or better threaded circular quick-connectors.

  - The interior of the VRC shall include steel strut–type wire enclosure tracks with cover strip to enclose wiring in its vertical run in an installed position.

- Power Source:  Owner shall terminate high voltage operating power within 10’ of the location designated for installation of the VRC.

2.04	FINISHES

- All carbon steel surfaces shall be coated with an industrial enamel finish over primer – color PFlow Blue.

- Prior to painting, all dirt, mill scale, oil, and grease shall be removed from carbon steel surfaces by a combination of brushing, wiping, and use of solvents.

PART 3 – EXECUTION

  - EXAMINATION

- Prior to commencing installation of the VRC, the installer shall visually examine the conditions under which the VRC is to be installed and notify the architect in writing of conditions detrimental to the proper and timely completion of the work.

  - INSTALLATION

- Install the VRC as indicated on the approved shop drawing.

- Comply with manufacturer’s detailed installation instructions when installing the equipment.

  - FIELD QUALITY CONTROL

- Inspection: Upon completion of installation, the VRC shall be inspected to verify that it meets all requirements of PARTS 1, 2, and 3 of this Section.

- Tests:

  - Operating Load Test:  The owner will provide a          pound test load and load the VRC at the ground level.  The loaded VRC platform shall be conveyed to an upper floor level and returned to the ground level to assure proper operation.  If the VRC conveyor cannot lift or lower the load, the VRC shall fail the test.

  - Performance Test:  This Test is to be performed in conjunction with Test 1 above. During the demonstration of the lifting and lowering test, the owner shall measure the time required to lift and lower the capacity load.  The owner will average times for lifting and lowering the load and calculate the average lifting and lowering speed.  If the VRC does not lift the load within 10% of the specified speed, or if the lowering speed exceeds the lifting speed by more than 10%, the VRC shall fail the test.

  - Stationary Load Test:  This Test is to be performed in conjunction with Test 1 above.  The loaded VRC platform shall remain stationary at an upper level for a minimum of one (1) hour.  After the one (1) hour period, the VRC will be inspected for deflection of the components or drift of the platform.  If deformation or downward drift is evident, the VRC shall fail the test.

  - ADJUSTING AND CLEANUP

- Touch up all scratches, abrasions, and other defects in the pre-finished surfaces with the same material color as that used in the factory applied finish.

- Remove and dispose of all rubbish and debris caused by the work under this section.

- Verify that equipment is properly installed and guarded per ASME B20.1

#### END OF SECTION